# Raw Data Reduction Template for WIRO DoublePrime

# Download the night's data into new directory

cd /crow1/reu/2018/iraf/

mkdir 20180104

cd 20180104

cp /d/hamlet2/Data.horatio/20180104/* .

# copy WDPzero.cl into working directory

cp /d/crow1/reu/2018/iraf/20180104/WDPzero.cl .

# zero-correct all images, remove overscan regions

task WDPzero=WDPzero.cl

ls -1 *.fit > raw.list

WDPzero("raw.list")

# create file with just the bias frames

ls b*_z.fits > bias.list

# combine overscan-subtracted bias images into a masterbias

imcombine input=@bias.list output=masterbias.fits combine=average reject=minmax

# create file containing names of all the _z zero-corrected images

ls *_z.fits > zero_out.list

# create bias_out.list, replacing suffix _z with _zb

!sed -e s/z.fits/zb.fits/ zero_out.list > bias_out.list

# create skyflat lists

hselect images=*_z.fits fields=$I,FILTER expr=yes > allFilter.list

!sed -ie s/z.fits/zb.fits/ allFilter.list

!grep "Filter 1" allFilter.list | cut -c1-12 > filter_g.list

!grep "Filter 2" allFilter.list | cut -c1-12 > filter_r.list

!grep "Filter 3" allFilter.list | cut -c1-12 > filter_u.list

!grep "Filter 4" allFilter.list | cut -c1-12 > filter_z.list

!grep "Filter 5" allFilter.list | cut -c1-12 > filter_i.list

# make sure to remove the bias frames, since technically they were taken with a certain filter

!nedit filter_* &

# create files with just the sky flats included

# filter files should have only science images now, and skyflats should have only flats

cp filter_g.list skyflats_g.list

cp filter_r.list skyflats_r.list

cp filter_u.list skyflats_u.list

cp filter_z.list skyflats_z.list

cp filter_i.list skyflats_i.list

!nedit skyflats_* filter_* &

# check to see if there are any garbage flat frames; remove from sky lists as needed.

task prevu=/d/crow1/tools/iraf/galphot/apr01/prevu.cl

prevu @zero_out.list

# subtract masterbias from every exposure

imarith @zero_out.list - masterbias.fits @bias_out.list

# combine flats of similar filter; combine the _zb.fits images

imcombine @skyflats_g.list masterflat_g.fits combine=average reject=avsigclip scale=median weight=median

imcombine @skyflats_r.list masterflat_r.fits combine=average reject=avsigclip scale=median weight=median

imcombine @skyflats_u.list masterflat_u.fits combine=average reject=avsigclip scale=median weight=median

imcombine @skyflats_z.list masterflat_z.fits combine=average reject=avsigclip scale=median weight=median

imcombine @skyflats_i.list masterflat_i.fits combine=average reject=avsigclip scale=median weight=median

# Normalize out masterflats to unity across the image

# Do the stats over the central regions, since the flats are not uniform

# and our galaxies basically reside in the image centers

imstat masterflat_*[1048:3048,1048:3048]

# update mean values for division

# create a normalized flat based on the mean level provided by imstat

imarith masterflat_g.fits / 37728 masterflat_g_norm.fits

imarith masterflat_i.fits / 52106 masterflat_i_norm.fits

imarith masterflat_r.fits / 21227 masterflat_r_norm.fits

imarith masterflat_u.fits / 41910 masterflat_u_norm.fits

imarith masterflat_z.fits / 54833 masterflat_z_norm.fits

# finally, divide all science images by the appropriate master normalized flat

!sed -e s/zb.fits/zbf.fits/ filter_g.list > science_g_f.list

!sed -e s/zb.fits/zbf.fits/ filter_r.list > science_r_f.list

!sed -e s/zb.fits/zbf.fits/ filter_u.list > science_u_f.list

!sed -e s/zb.fits/zbf.fits/ filter_z.list > science_z_f.list

!sed -e s/zb.fits/zbf.fits/ filter_i.list > science_i_f.list

imarith @filter_g.list / masterflat_g_norm.fits @science_g_f.list

imarith @filter_r.list / masterflat_r_norm.fits @science_r_f.list

imarith @filter_u.list / masterflat_u_norm.fits @science_u_f.list

imarith @filter_z.list / masterflat_z_norm.fits @science_z_f.list

imarith @filter_i.list / masterflat_i_norm.fits @science_i_f.list

# cleanup processing files to save memory

rm -f *.fit

rm -f *_z.fits

rm -f *_zb.fits